## Artesania Latina  MayFlower

## 1

From the sheets marked "A", remove the outer keel (1), the frames (2 to 13), and the stiffeners (14 and 19). Locate and glue the frames (2 to 13), in their corresponding locations on the outer keel (1). Locate and glue the upper stern, frame, outer keel and mast stiffeners, following the numerical order of the photo, using wood adhesive.

## 2

From sheet "B" remove the decks (20, 21,22, 23, 25, 27, 29 and 31). Decks (20, 21, and 22) are varnished and are located as indicated in the photo. Decks (23, 25, 27, 28, 29 and 31) must be lined with 0.6 x 5 mm. ramon wood planking sheets. In continuation varnish the decks. Once they have dried finish the operation fitting and gluing the decks to their corresponding locations.

## 3-4

Line the top part of the frame (5) with the mahogany 0.6 x 5 mm. planking sheets (33). Carry out the same operation with the bulkheads (34, 36 and 38) with the mahogany planking sheets (35, 37 and 39). Finally glue the poop deck (31) in place.

## 5-8

Taking the utmost care sand down the outer profile of the frames and stiffeners, from the bow towards the centre of the hull, in order to achieve a good base, which will allow for the location at a later stage of the planking strakes (40). Carry out the same operation working from the stern. We will realism that the more curvature the hull has, the more wood we will sand off.

Very important: When locating the strakes, be careful not to press them in too hard, as the line of a hull is uneven, we suggest it should be done in the following way.

Glue and pin a strake (40) along the full length of the hull, starting off three millimeters from the upper part of the central frames (6 and 9). Do the same on the other side of the hull. Successively and alternatively cover the hull, that is for each strake located along one side, place its equal on the other side of the hull. In continuation glue and pin another strake below the first, in such a way that the edges are in contact with each other. Use would adhesive and the pins (41). Continue covering the keel, working from the bottom towards the deck. Finally, in the middle of the hull, the gaps which remain are covered with sections of strakes, sharpened at both ends, called fillers. At the bow and the stern the covering is done with wedges sharpened at one end.

## 9

From sheet "C" remove the bulwarks (42) so as to line them on the inside with the mahogany planking sheet (43). In continuation varnish the inside of the bulwarks. Once they are dry glue and pin the bulwarks from bow to stern, seating them along the edge of the top strake and against the frames (see the plan drawings).

## 10

From sheet "B", remove the upper stern (44) and glue it to the back of the bulwarks (42). In continuation line the whole of the stern (except the stiffener (14) and the upper stern (44), see photo 1) using the planking strakes (40). Once the whole of the hull has been covered, we sand down the surface until we have a smooth and even finish. Also sand down the profile where the stern (45), the forefoot (46), the keel (47-48) and the post (49) have to be glued, until you achieve the necessary 5 mm. flat emplacement. (Remove the parts from sheet "C").

## 11

Using contact adhesive (see general advice) line the stern with the mahogany planking sheets (50). Carry out the same operation with the outside of the bulwarks, working from the bottom upwards. Finally glue the stem rake (51) and the braces (52).

## 12

On the basis of 1.5 x 3 mm. strips of African walnut, make up part (53). Go ahead with the same operation using the 2 x 2 mm. strips of African walnut, making up the parts (54 and 56).

## 13

With 1.5 x 3 mm. strips of African walnut, make up the parts (57, 59, 60, 61, 62, 63, 64 and 65). Glue them in place following the plan drawings.

## 14

With 2x5 mm. strips of African walnut, make up all of the gunwale plank sheer (66, 67, 68, 70, 71 and 72). From sheet "C", remove curved fore ends (69). Fit and glue these parts in place, following the plan drawings.

## 15

Before going ahead with this stage of the assembly, carefully examine the four boxes which make up photo 15. Following the numerical order, and using the parts list as a guide, make up and glue all of the parts required at this stage of the assembly.

## 16-32

The most delicate and laborious work has now been done. From photo 16 onwards it is a question of making up and locating the parts step by step according to the sequence of photographs, using the parts list and the plan drawings as a guide. Looking at the diagrams of the masts on the plan, you will see that all of the masts are conical. To achieve this conical shape take the measurements from the masts diagram. Once this has been done, start to sand the mast, from large to small, to give it the correct shape [use coarse grain sandpaper]. Mount the parts which make up the masts, without their tackle [straps, blocks, eye bolts, etc.] and varnish them. In continuation assemble the tackle with all the components, as indicated in the plans, including the gaffs and booms etc. Once this work has been done, locate and glue all of the masts in place, as indicated in the plans.

From this point on you can begin to make up the rigging, the mobile and fixed part. Given the many ropes of which the rigging consists, we believe that the explanation relating to each of them could lead to confusion. For this reason we have attached a series of simplified diagrams, to the overall plans, where the arrangement of all of the rigging can be seen, step by step [see figures from the plan]. To make up the sails follow these steps:

Using the pattern included in the kit, cut and hem the sails. Sew the [0.8 mm. diameter cotton thread] sail bolt-rope (316) around the whole perimeter of the sail, leaving an eye at all the corners, where the ropes [lines] can later be tied. Read "General Advice".

We are sure that you will successfully finish this attractive model

ENJOY IT!