### ELEMENT! NUMERATI SULLE TAVOLE

### NUMBERED ELEMENTS ON THE PLATES

### PIECES NUMEROTEES SUR LES PLANCHES

### NUMERIERTE TEILE AUF DEN TAFELN

| N. No. N. Nr. | Denominazione Name Designation Bezeichnung | Materiale Material Matiere Material | Misure Measures Mesures Mape | N. Cat. No. Cat. N. Cat. Nr. Kat. | Sigla Symbol Sigle Sigel |
| --- | --- | --- | --- | --- | --- |
| 1 | Chiglia Keel Quille Kiel | compensato plywood C.T.P. Sperrholz | 5 mm |  | P |
| 2 | Riempitore prua Bow filler Remplisseur de proue Bugfullklotze | compensato plywood C.T.P. Sperrholz | 4 mm |  | P |
| 3/14 | Ordinate Frames Couples Spanten | compensato plywood C.T.P. Sperrholz | 4 mm |  | P |
| 15 | Piano poppa Stern plane Plan de poupe Heckplane | compensato plywood C.T.P. Sperrholz | 4 mm |  | P |
| 16 | Specchio poppa inferiore Lower transom Tableau de poupe inferieur Unterheckspiegel | compensato plywood C.T.P. Sperrholz | 4 mm |  | P |
| 17 | Specchio poppa superiore Upper transom Tableau de poupe superieur Oberheckspiegel | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 18 | Ponte prua Bow deck Pont de proue Bugdeck | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 19 | Castello Forecastle Pont du gaillard d'avant Backdeck | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 20 | Coperta Main deck Pont principal Grossdeck | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 21 | Mezzoponte Half-deck Demi-pont Halbdeck | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 22 | Cassero Quarter deck Pont du gaillard d'arriere Kampanjedeck | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 23 | Casseretto Poop deck Pont de la dunette Puppdeck | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 24 | Paratia prua Bow bulkhead Cloison de proue Bugschotte | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 25 | Paratia mezzoponte Half-deck bulkhead Cloison du demi-pont Halbdeckschotte | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 26 | Riempitore poppa Stern filler Remplisseur de poupe Heckfullklotze | massello chock bloc Block | 90 mm |  | P |
| 27 (a - b - c - d - e - f - g - h - i - 1) |  |  |  |  |  |
|  | 1° fasciame 1st planking 1er borde 1. Beplankung | listello tiglio lime board baguette tilleul Lindenholzleiste | 1,5 x 5 mm | LS220 | R |
| 28 | Supporto canne tronche Half-guns support Supports des canons a piquer Halbrohreplatte | list, tiglio a "U" "U" lime board baguette tilleul "U” Lindenholzleiste “U” | 8x12 mm |  | R |
| 29 | Supporto canne tronche Half-guns support Supports des canons a piquer Halbrohreplatte | list, tiglio a "U" "U" lime board baguette tilleul "U” Lindenholzeiste “U” | 10x14 mm |  | R |
| 30 | Galleria poppa Quarter gallery Bouteille d'arriere Heckgalerie | massello chock bloc Block | 55 mm |  | P |
| 31 | Fasciame superiore poppa Stern upper planking Revetement superieur d'arriere Heckoberbeplankung | listello noce walnut board baguette noyer Nussholzleiste | 1x3 mm | LS230 | R |
| 32 | Fasciame inferiore poppa Stern lower planking Revetement inferieur d'arriere Heckunterbeplankung | listello noce walnut board baguette noyer Nussholeleiste | 1x3 mm | LS230 | R |
| 33 | 2° fasciame-1a cinta 2nd planking-1st wale 2eme borde-1ere ceinte 2. Beplankung-1. Barkholz | listello noce walnut board baguette noyer Nussholzleiste | 2x3 mm | LS235 | R |
| 34 | 2° fasciame-1a banda 2nd planking-1st strip 2eme borde-1ere bande 2. Beplankung-1. Streif | listello noce walnut board baguette noyer Nussholzleiste | 1x3 mm | LS230 | R |
| 35 | 2° fasciame-2a cinta 2nd planking-2nd wale 2eme borde-2eme ceinte 2. Beplankung-2. Barkholz | listello noce walnut board baguette noyer Nussholzleiste | 2x3 mm | LS235 | R |
| 36 | 2° fasciame-2a banda 2nd planking-2nd strip 2eme borde-2eme bande 2. Beplankung-2. Streif | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 37 | 2° fasciame-3a cinta 2nd planking-3rd wale 2eme borde-3eme ceinte 2. Beplankung-3. Barkholz | listello noce walnut board baguette noyer Nussholzleiste | 2x3 mm | LS235 | R |
| 38 | 2° fasciame-3a banda 2nd planking-3rd strip 2eme borde-3eme bande 2. Beplankung-S.Streif | listello noce walnut board baguette noyer Nussholzleiste | 1x3 mm | LS230 | R |
| 39 | 2° fasciame-4a cinta 2nd planking-4th wale 2eme borde-4eme ceinte 2. Beplankung-4. Barkholz | listello noce walnut board baguette noyer Nussholzleiste | 2x3 mm | LS235 | R |
| 40 | 2° fasciame-4a banda 2nd planking-4th strip 2eme borde-4eme bande 2. Beplankung-4. Streif | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 41 | 2° fasciame-5a cinta 2nd planking-5th wale 2eme borde-5eme ceinte 2. Beplankung-5. Barkholz | listello noce walnut board baguette noyer Nussholzleiste | 2x3 mm | LS235 | R |
| 42 | 2° fasciame-5a banda 2nd planking-5th strip 2eme borde-5eme bande 2. Beplankung-5. Streif | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 43 | 2° fasciame-incintone 2nd planking-main wale 2eme borde-preceinte 2. Beplankung-Grossbarkholz | listello noce walnut board baguette noyer Nussholzleiste | 2x4 mm | LS236 | R |
| 44 | 2° fasciame-zona inferiore 2nd planking-lower area 2eme borde-partie inferieure 2. Beplankung-Unterteil | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 45 | 2° fasciame-zona superiore centrale 2nd planking-central upper area 2eme borde-partie superieure centrale 2. Beplankung-Obermittelteil | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 46 | 2° fasciame-zona superiore prua 2nd planking-bow upper area 2eme borde-partie superieure de proue 2. Beplankung-Bugoberteil | listello noce walnut board baguette noyer Nussholzleiste | 0,6x4 mm | LS225 | R |
| 47 | 2° fasciame-zona superiore poppa 2nd planking-stern upper area 2eme borde-partie superieure d'arriere 2. Beplankung-Heckoberteil | listello noce walnut board baguette noyer Nussholzleiste | 0,6x4 mm | LS225 | R |
| 48 | Battuta portelli Ports-post Montant des sabords Pfortenpfosten | listello faggio beech board baguette hetre Buchenholzleiste | 1 x2mm |  | R |
| 49 | Fasciame-interno murata Bulwark inner planking Revetement interieur du pavois Schanzkleidinnerbeplankung | listello noce walnut board baguette noyer Nussholzleiste | 0,6x3 mm |  | R |
| 50 | Fasciame paratia N.'25 e settori ordinate Bulkhead No. 25 and frames area planking Borde du cloison N. 25 et des secteures des couples Short Nr. 25 und Spantesektor Beplankung | listello noce walnut board baguette noyer Nussholzleiste | 0,6x3 mm |  | R |
| 51 | Fasciame ponti - margine corsia Decks planking - midway edge Borde des ponts - cc-te de la coursie Deckbeplankung -Mittelgangseite | listello noce walnut board baguette noyer Nussholzleiste | 1 x4 mm | LS231 | R |
| 52 | Fasciame ponti - corsia Decks planking - midway Bord6 des ponts - coursie Deckbeplankung - Mittelgang | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 53 | Fasciame ponti Decks planking Borde des ponts Deckbeplankung | list, tanganica tanganika board baguette tanganika Tanganikaholzleiste | 0,6x3 mm | LS244 | R |
| 54 | Trincarino Waterway Fourrure de gouttiere Wassergang | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 55 | Contorno paratia N. 25 e settori ordinate Bulkhead No. 25 and frames area contour Contour du cloison N. 25 et des secteurs des couples Shott Nr. 25 und Spantensektorumrip | listello noce walnut board baguette noyer Nussholzleiste | 1 x1 mm | LS228 | R |
| 56 | Falso scalmotto False toptimber Fausse allonge de sommet Falschstiitze | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 57 | Falsa cinta False wale Fausse ceinte Falschbarkholz | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 58 | Falso scalmotto - settore sup. False toptirnber - upper part Fausse allonge de sommet -partie superieure Falschstutze - Oberteil | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 59 | Rinforzo aperture cannoni Guns openings reinforcement Renfort des ouvertures des canons Kanonenoffnungverstarkung | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 60 | Falso dormiente False clamp Fausse bauquiere Falschbalkweger | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 61 | Cinta interna Inner wale Ceinte interieure Innerbarkholz | listello faggio beech board baguette hetre Buchenholzleiste | 1x1 mm |  | R |
| 62 | Contorno ponte N. 18 Deck No. 18 countour Contour du pont N. 18 Deck Nr. 18 Umrip | listello noce walnut board baguette noyer Nussholzleiste | 0,6 x 3 mm |  | R |
| 63 | Rinforzo murata prua Bow bulwark reinforcement Renfort du pavois de proue Bugschanzkleidverstarkung | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 64 | Fasciame paratia N. 24 Bulkhead No. 24 planking Borde du cloison N. 24 Schott Nr. 24 Beplankung | listello noce walnut board baguette noyer Nussholzleiste | 0,6x4 mm | LS225 | R |
| 65 | Capodibanda Planksheer Lisse de pavois Schanzkleidreling | listello faggio beech board baguette hetre Buchenholzleist* | 1x4 mm |  | R |
| 66 | Corrimano murata Bulwark handrail Main courante du pavois Schanzkleidhandlauf | listello faggio beech board baguette hetre Buchenholzleiste | 1x1 mm |  | R |
| 67 | Raccordo prua Bow connection Raccord de proue Bugverbindung | listello noce walnut board baguette noyer Nussholzleiste | 1x3 mm | LS230 | R |
| 68 | Margine fasciame prua Bow planking edge Contour du borde de proue Bugbeplankungumrip | listello faggio beech board baguette hetre Buchenholzleiste | 1x2 mm |  | R |
| 69 | Carabottino Grating Caillebotis Grating | legno wood bois Holz |  | P10 | P |
| 70 | Contorno carabottini (incassato) Gratings contour (embedded) Contour des caillebotis (encaisse) Gratingurmirp (einsparung) | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 71 | Contorno carabottini (non incassato) Gratings contour (not embedded) Contour des caillebotis (non encaisse) Gratingumrip (nicht einsparung) | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 72 | Contorno boccaporto principale e aperture manovre (incassato) Main hatch and rigging openings contour (embedded) Contour de I'ecoutille principale et des ouvertures des manoeuvre (encaisse) Grossluke und Tauwerkoffnung Umrip (einsparung) | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 73 | Supporto (incassato) Support (embedded) Support (encaisse) Basis (einsparung) | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 74 | Quartiere boccaporto principale Main hatch covering Panneau de I'ecoutille principale Grosslukendekel | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 75 | Mastra argano Capstan bed plate Etambrai de cabestan Gangspillgrundplatte | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 76 | Argano Capstan Cabestan Gangspill | legno wood bois Holz |  | A 144 | A |
| 77 | Ascialone argano Capstan whelp Tasseau de cabestan Gangspillrippe | legno wood bois Holz |  | A 144 | A |
| 78 | Scontro argano Capstan pall Linguet de cabestan Gangspillpallen | listello faggio beech board baguette hetre Bechuenholzleiste | 1 x2 mm |  | R |
| 79 | Chiodo Nail Clou Nagel | metallo metal metal Metall |  | C150 | A |
| 80 | Aspa argano Capstan bar Barre de cabestan Gangspillspaken | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 m |  | R |
| 81 | Piede alberi Masts foot Pied des mats Mastfup | listello noce walnut board baguette noyer Nussholzleiste | 2x4 mm | LS236 | R |
| 82 | Quartiere boccaporti minori Little hatches covering Panneau des ecoutilles petites Kleinenlukendeckel | listello noce walnut board baguette noyer Nussholzleiste | 2x3 mm | LS235 | R |
| 83 | Contorno boccaporti minori e aperture manovre Little hatches and rigging opening contour Contour des ecoutilles petites et des ouvertures des manoeuvres Kleinenluken und Tauwerkoffnungumnrig | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 84 | Bitta cavigliera Belaying rack bitt Bitte du ratelier Nagelbankbeting | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 85 | Protome bitte Head for bitts Tete pour les bittes Betingkopf | fusione metallo dorato gilded cast metal fusione metal doire Metallguss Vergoldete |  | D825 | A |
| 86 | Contorno bitte Bitts contour Contour des bittes Betingumrip | listello faggio beech board baguette hetre Buchenholzleiste | 1 x1 mm |  | R |
| 87 | Tavola cavigliera Belaying rack Lisse du ratelier Nagelbank | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm * |  | R |
| 88 | Bittone Knighthead Bitton Knecht | listello faggio beech board baguette hetre Buchenholzleiste | 5x5 mm i |  | R |
| 89 | Bitta Bitt Bitte Beting | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm i | LS256 | R |
| 90 (a b c) |  |  |  |  |  |
|  | Cavigliera murata Bulwark belaying rack Ratelier du pavois Schanzkleidnagelbank | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm | - | R |
| 91 | Camino Chimney Cheminee Kamin | ottone fotoincisc photoetched brass laiton photograve Photogravurmesing |  | D40 | A |
| 92 | Contorno camino Chimney contour Contour de la cheminee Kaminumrip | listello faggio beech board baguette hetre Buchenholzleiste | 1 x1 mm |  | R |
| 93 | Anta porte e finestre Doors and windows shutter Ante des portes et des fenetres Tur und Fensterante | listello faggio beech board baguette hetre Buchenholzleiste | 1 x2 mm |  | R |
| 94 | Cornice porte paratia N. 24 Bulkhead No. 24 doors squaring Corniche des portes du cloison N. 24 Schott Nr. 24 Turrahmen | ottone fotoincisc photoetched brass Iaiton photograve Photogravurmesing |  | D40 | A |
| 95 | Falsa cerniera False hinge Fausse charniere Falschscharniere | ottone brass Iaiton Messing |  | (S 150) | A |
| 96 | Bordura paratia N. 24 Bulkhead No. 24 bordure Bordure du cloison N. 24 Schott Nr. 24 Bordure | ottone fotoincisc photoetched brass Iaiton photograve Photogravurmesing |  | D40 | A |
| 97 | Blasone paratia N. 24 Bulkead No. 24 coat of arm Blason du cloison N. 24 Schott Nr. 24 Wappen | ottone fotoincisc photoetched brass Iaiton photograve Photogravurmesing |  | D40 | A |
| 98 | Cariatide paratia N. 24 Bulkhead No. 24 caryatid Cariatide du cloison N. 24 Schott Nr. 24 Kariatide | fusione metallo dorato gilden cast metal fusione metal dore Metalguss Vergoldete |  | D828 | A |
| 99 | Cornice porte ordinata N. 6 Frame No. 6 doors squaring Corniche des portes de la couple N. 6 Spant Nr. 6 Turumrip | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 100 | Cornice porte ordinata N. 12 Frame No. 12 doors squaring Corniche des portes de la couple N. 12 Spant Nr. 12 Turumrip | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 101 | Falsa grata False grate Fausse grille Falschfachwerk | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 102 | Riempitore porte paratia N. 25 Bulkhead No. 25 doors filler Remplisseur des portes du cloison N. 25 Schott Nr. 25 Turfiillklotze | tondino ramino 0 10 mm ramino round rond ramino Raminorund |  | TO 10 | R |
| 103 | Cornice porte paratia N. 25 Bulkhead No. 25 doors squaring Corniche des portes du cloison N. 25 Schott Nr. 25 Turumrip | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 104 | Falso colonnato False colonnade Fausse colonnade Falschkolonnade | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 105 | Cartiglio Cartouche Cartouche Schriftrolle | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 106 | Balaustra Balustrade Balustrade Balustrade | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 107 | Arcata Arcade Arcade Arkade | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 108 | Cariatide cassero Quarter deck caryatid Cariatide du pont du gaillard d'arriere Kampanjedeckkaryatide | fusione metallo dorato gilded cast metal fusione metal dore Metallguss Vergoldete |  | D829 | A |
| 109 | Cornice porta ordinata N. 13 Frame No. 13 door squaring Corniche de la porte de la couple N. 13 Spant Nr. 13 Turumrip | ottone fotoinciso photoetched brass laiton photograve Photograviirmessing |  | D40 | A |
| 110 | Feritoia ordinata N. 13 Frame No. 13 embrasure Soupirail de la couple N. 13 Spant Nr. 13 Spalt | ottone brass laiton Messing |  |  | A |
| 111 | Riempitore basi battagliole Railings bases filler Remplisseur des bases des batayoles Relingbasisfullklfltze | list, tanganica tanganika board baguette tanganika Tanganikaholzleiste | 0,6 x 3 mm | LS244 | R |
| 112 | Base battagliole Railings base Base des batayoles Relingbasis | listello faggio beech board baguette netre Buchenholzleiste | 1 x3 mm |  | R |
| 113 | Supporto battagliole Railings stanchion Montant des batayoles Relingstiitze | listello faggio beech board baguette hetre Buchenholzleist | 2x2 mm |  | R |
| 114 (a - b) |  |  |  |  |  |
|  | Corrimano battagliole Railings top plank Main courante des batayoles Relinghandlauf | listello faggio beech board baguette hetre Buchenholzleiste | 1 x3 mm |  | R |
| 115 | Fregio battagliola castello Forecastle railing frieze Frise de la batayole du pont du gaillard d'avant Backdeckrelingverzierung | ottone fotoinciso photoetched brass laiton photograve Photograviirmessing |  | D40 | A |
| 116 | Fregio battagliola cassero Quarter deck railing frieze Frise de la batayole du pont du gaillard d'arriere Kampanjedeckrelingverzierung | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 117 | Blasone battagliola cassero Quarter deck railing coat of arms Blason de la batayole du pont du gaillard d'arriere Kampanjedeckrelingwappen | ottone fotoinciso photoetched brass laiton photograve Photogravii rmessi ng |  | D40 | A |
| 118 | Fregio battagliola casseretto Poop deck railing frieze Frise de la batayole du pont de la dunette Puppdeckrelingverzierung | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 119 | Colonnina Column Colonnette Saiile | legno wood bois Holz |  | C65 | A |
| 120 | Contorno passerelle Marines walks countour Contour des passerelles Laufstegumrip | listello faggio beech board baguette hetre Buchenholzleiste | 1 x2 mm |  | R |
| 121 | Contorno ponti Decks contour Contour des ponts Decksumrip | listello faggio beech board baguette hetre Buchenholzleiste | 1x1 mm |  | R |
| 122 | Suola bitte a orecchie Staghorns sole Semelle des taquets a oreilles Kreuzholzbasis | listello faggio beech board baguette hetre Buchenholzleiste | 1 x3 mm |  | R |
| 123 (a - b - c - d) |  |  |  |  |  |
|  | Bitta a orecchie Staghorn Taquet a oreilles Kreuzholz | massello chock block Block | 2 mm |  | P |
| 124 | Fianco scale Ladders Cote des echelles Treppenwange | legno wood bois Holz |  |  | P |
| 125 | Gradino scale Ladders step Qradin des echelles Treppenstufe | listello noce walnut board baguette noyer Nussholzleiste | 0,6x3 mm |  | R |
| 126 | Montante scala casseretto Poop deck ladder pillar Montant de I'echelle du pont de la dunette Puppdecktreppestange | listello faggio beech board baguette hetre Buchenholzleiste | 1 x2 mm |  | R |
| 127 | Gradino sagomato Mouled step Gradin galbe Profilierstufe | list, faggio "L" beech board "L" beguetta hetre "L” Buchenholzleiste “L” | 2x2 mm |  | R |
| 128 | Fasciame cubie Hawseholes planking Defense des ecubiers Ankerklusenbeplankung | listello noce walnut board baguette noyer Nussholzleiste | 1 x4 mm | LS231 | R |
| 129 | Sperone Knee of the head Eperon Scheg | compensato plywood C.T.P. Sperrholz | 5 mm |  | P |
| 130 | Freccia Extension piece Fleche Auslieger | compensato plywood C.T.P. Sperrholz | 5 mm |  | P |
| 131 | Fregiata Frieze of the head Frize de I'eperon Schegkamm | fusione metallo dorato glided cast metal fusione metal dore Metallguss Vergoldete |  | D822 | A |
| 132 | Bracciolo Knee Courbe Knie | massello chock bloc Block | 3 mm |  | P |
| 133 | 1a curva 1st cheek rail 1er dauphin 1. Schloiknieregel | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 3x3 mm | LF303 | R |
| 134 | 2a curva 2nd cheek rail 2 erne dauphin 2. Schloiknieregel | list . faggio flex, beech board flex baguette hetre fliex Buchenholzleiste flex | 3x3 mm | LF303 | R |
| 135 | Polena Figure-head Figure de proue Galionsfigur | fusione metallo dorato gilded cast metal fusione metal dore MetallguSs Vergoldete |  | D820 | A |
| 136 | Voluta Volute Volute Volute | fusione metallo dorato gilded cast metal fusione metal dore Metallguss Vergoldete |  | D821 | A |
| 137 | Struttura serpe Head structure Structure de la herpe Galionsstruktur | ottone fotoinciso photoetched brass laiton photograve Photograviirmessing |  | D40 | A |
| 138 | Traversa cani serpe Head timbers beam Traverse des jambettes de la herpe Galionsspantenbalken | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 139 | Scalmotto cani serpe Head timbers stranchion Allonge des jambettes de la herpe Galionsspantenstutze | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 140 | Carabottino serpe Head grating Caillebotis de la herpe Galionsgrating | legno wood bois Holz |  | P10 | P |
| 141 | Contorno carabottini Gratings contour Contour des caillebotis Gratingsumrip | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 142 | Contorno carabottini Gratings contour Contour des caillebotis Gratingsumrip | listello faggio beech board baguette hetre Buchenholzleiste | 1 x3 mm |  | R |
| 143 | Latrina Latrines Poulaines Latrine | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 144 | Supporto latrine Latrines support Support des poulaines Latrinesbasis | listello faggio beech board baguette hetre Buchenholzleiste | 1x3 mm |  | R |
| 145 | 1a serpe 1st head rail 1ere herpe 1. Galionsregel | list, faggio flex, beech board flex baguete hetre flex Buchenholzeiste flex | 3x3 mm | LF303 | R |
| 146 | 2a serpe 2nd head rail 2eme herpe 2. Galionsregel | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 3x3 mm | LF303 | R |
| 147 | 3a serpe 3rd head rail Seme herpe 3. Galionsregel | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 3x3 mm | LF303 | R |
| 148 | Riempitore cani serpe Head timbers filler Remplisseuir des jambettes de la herpe Galionsspantenfiillklotze | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 149 | Cariatide cani serpe Head timbers caryatid Cariatide des jambettes de la herpe Galionsspantenkaryatide | fusione metallo dorato gilded cast metal fusione metal dore Metallguss Vergoldete |  | D830 | A |
| 150 | Protome serpe Head top decoration Tele de la herpe Galionskopf | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D823 | A |
| 151 | Fregio serpe Head rails frieze Frise de la herpe Galionsregelverzierung | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 152 | Cavigliera serpe Head belaving rack Ratelier de la herpe Galionsnagelbank | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 153 | Gru capone Cathead Bossoir de capon Kranbalken | listello faggio beech board baguette hetre Buchenholzleiste | 4x4 mm | LS257 | R |
| 154 | Puleggia capone Cathead sheave Rea des bossoirs de capon Kranbalkenscheide | ottone brass Iaiton Messing | d 4 mm | P50 | A |
| 155 | Cariatide capone Cathead caryatid Cariatide des bossoirs de capon Kranbalkenkaryatide | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D827 | A |
| 156 | Protome murata Bulwark head Tete du pavois Schanzkleidkopf | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D824 | A |
| 157 | Cornice sabordi Round-ports decoration Corniche des sabords Kanonenoffnungrahmen | ottone fotoinciso photoetched brass Iaiton photograve Photogravurmessing |  | D40 | A |
| 158 | Feritoia murata Bulwark embrasure Soupirail du pavois Schanzkleidspalt | ottone brass Iaiton Messing |  |  | A |
| 159 | Cariatide passacavo Kevel caryatid Cariatide avec conduit des manoeuvres Karyatide mit Klampen | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D826 | A |
| 160 (a- b-c-d - e-f -g) |  |  |  |  |  |
|  | Feritoia passacavo Kevel embrasure Soupirail avec conduit des manoeuvres Spalt mit Klampen | ottone brass Iaiton Messing |  |  | A |
| 161 | Ombrinale Scupper Dalot Speigatt | ottone fotoinciso photoetched brass Iaiton photograve Photogravurmessing |  | D40 | A |
| 162 | Finestra poppa Stern window Fenetre d'arriere Heckfenster | ottone fotoinciso photoetched brass Iaiton photograve Photogravurmessing |  | D40 | A |
| 163 | Riempitore finestre gallerie Galeries windows filler Remplisseur des fenetres des bouteilles Galerienfensterfullklotze | massello chock bloc Block | 15 mm |  | P |
| 164 | Fasciame inferiore gallerie Galleries lower planking Borde inferieur des bouteilles Galerienunterbeplankung | listello noce walnut board baguette noyer Nussholzleiste | 0,6x3 mm |  | R |
| 165 | Fasciame superiore gallerie Galleries upper planking Borde superieur des bouteilles Galerienoberbeplankung | listello noce walnut board baguette noyer Nussholzleiste | 0,6x4 mm | LS225 | R |
| 166 | Spigolo gallerie Galeries edge Arete des bouteilles Galerienkante | Iistello faggio beech board baguette hetre Buchenholzleiste | 1 x1 mm |  | R |
| 167 | Bordura gallerie Galleries bordure Bordure des bouteilles Galerienbordiire | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 168 | Cornice finestre gallerie Galleries windows squaring Corniche des fenetres des bouteilles Galerienfensterrahmen | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 169 | Timone Rudder Gouvernail Ruder | compensato plywood C.T.P. Sperrholz | 5 mm |  | P |
| 170 | Rinforzo timone Rudder reinforcement Renfort du governail Ruderverstarkung | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 171 | Cerniera timone Rudder timone Chaniere du governail Ruderscharniere | metallo metal metal Metall |  | C51 | A |
| 172 | Spina cerniere timone Rudder hinges pin Cheville des charnieres du gouvernail Ruderscharnierenpflock | metallo metal metal Metall |  | C51 | A |
| 173 | Barra timone Rudder tiller Barre du gouvernail Ruderpinne | Iistello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 174 | Chiave barra timone Rudder tiller fid Ckef de la barre du gouvernail Ruderspinneschlopholz | Iistello faggio beech board baguette hetre Buchenholzleiste | 1 x2 mm |  | R |
| 175 | Passacavo trozza timone Rudder slings kevel Conduit des sauvegardes du gouvernail Rudersorgleinenklampen | Iistello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 176 | Raccordo specchio poppa Transom connection Raccord du tableau de poupe Heckspiegelverbindung | Iistello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 177 | 1a modanatura specchio poppa 1st transom moulding 1ere moulure du tableau de poupe 1. Heckspiegelsims | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 2x5 mm | LF205 | R |
| 178 | 2a modanatura specchio poppa 2nd transom moulding 2eme moulure du tableau de poupe 2. Heckspiegelsims | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 2x2 mm | LF202 | R |
| 179 | 3a modanatura specchio poppa 3rd transom moulding Seme moulere du tableau de poupe 3. Heckspiegelsims | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 2x2 mm | LF202 | R |
| 180 | 4a modanatura specchio poppa 4th transom moulding 4eme moulure du tableau de poupe 4. Heckspiegelsims | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 2x2 mm | LF202 | R |
| 181 | Margine laterals specchio poppa Transom side contour Contour lateral du tableau de poupe Heckspiegelseitenumrip | iistello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm | — | R |
| 182 | 5a modanatura specchio poppa 5th transom moulding Seme moulure du tableau de poupe 5. Heckspiegelsims | Iistello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 183 | 6a modanatura specchio poppa 6th transom moulding 6eme moulure du tableau de poupe 6. Heckspiegelsims | Iistello noce walnut board baguette noyer Nussholzleiste | 1 x 4 mm | LS231 | R |
| 184 | Coronamento specchio poppa Transom taffrail Couronnement du tableau de poupe Heckspiegelkronung | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D831 | A |
| 185 | Blasone coronamento specchio poppa Transom taffrail coat of arms Blason pour le couronnement du tableau de poupe Heckspiegelkronungwappen | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 186 | Fregio superiore specchio poppa Transom upper frieze Frise sup6rieur du tableau de poupe Heckspiegeloberverzierung | ottone fotoinciso photoetched brass laiton photograve Photograviirmessing |  | D40 | A |
| 187 | Modanatura fregio superiore specchio poppa Transom upper frieze moulding Moulure sur le frise superieur du tableau de poupe Heckspiegeloberverzierungsims | listello faggio beech board baguette hetre Buchenholzleist | 1 x1 mm |  | R |
| 188 | Putto specchio poppa Transom putto Petit amour du tableau de poupe Heckspiegelamorette | fusione metallo dorato gilded cast metal fusion m6tal dor6 Metallguss Vergoldete |  | D834 | A |
| 189 | Figura specchio poppa Transom figure Figure du tableau de poupe Heckspiegelfigur | fusione metallo dorato gilded cast metal fusion m6tal dor6 Metallguss Vergoldete |  | D832 | A |
| 190 | Leone specchio poppa Transom lion Lion du tabelau de poupe Heckspiegelldwe | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D833 | A |
| 191 | Drappeggio specchio poppa Transom draping Drapage du tableau de poupe Heckspiegeldraperie | fusione metallo dorato gilded cast metal fusion metal dor6 Metallguss Vergoldete |  | D835 | A |
| 192 | Mensola centrale specchio poppa Transom central corbel Console centrale du tableau de poupe Heckspiegelmittelkonsole | fusione metallo dorato gilded cast metal fusion m§tal dor6 Metallguss Vergoldete |  | D836 | A |
| 193 | Mensola specchio poppa Transom corbel Console du tableau de poupe Heckspiegelkonsole | fusione metallo dorato gilded cast metal fusion m6tal dor6 Metallguss Vergoldete |  | D837 | A |
| 194 | Finestratura specchio poppa Transom windows Fen^tres du tableau de poupe Heckspiegelfenster | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 195 | Cariatide piccola specchio poppa Transom small caryatid Petite cariatide du tableau de poupe Heckspiegelkleinkaryatide | fusione metallo dorato gilded cast metal fusion m6tal dor6 Metallguss Vergoldete |  | D838 | A |
| 196 | Fregio laterale specchio poppa Transom side frieze Frise lateral du tableau de poupe Heckspiegelseitenverzierung | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 197 | Bordura specchio poppa Transom bordure Bordure du tableau de poupe Heckspiegelbordure | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 198 | Fregio inferiore specchio poppa Transom lower frieze Frize inferieur du tableau de poupe Heckspiegelu nterverzieru ng | ottone fotoinciso photoetched brass laiton photograv6 Photogravurmessing |  | D40 | A |
| 199 | Blasone inferiore specchio poppa Transom lower coat of arms Blason inferieur du tableau de poupe Heckspiegelunterwappen | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | A |
| 200 | Cariatide grande specchio poppa Transom great caryatid Grande cariatide du tableau de poupe Heckspiegelgrosskaryatide | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D839 | A |
| 201 | Modanatura fregio inferiore specchio poppa Transom lower frieze moulding Moulure sur le frise inferieur du tableau de poupe Heckspiegelunterverzierungsims | listello faggio beech board baguette hgtre Buchenholzleiste | 1 x1 mm |  | R |
| 202 | Cariatide curva specchio poppa Transom curved caryatid Cariatide courb6 du tableau de poupe Heckspiegelkrummkaryatide | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D840 | A |
| 203 | Lesena specchio poppa Transom pilaster Pilastre du tableau de poupe Heckspiegellisene | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D841 | A |
| 204 | Figura gallerie Galleries figure Figure des bouteilles Galerienfigur | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D842 | A |
| 205 | Delfino gallerie Galleries dolphin Dauphin des bouteilles Galeriendelphin | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D843 | A |
| 206 | Cariatide gallerie Galleries caryatid Cariatide des bouteilles Galerienkaryatide | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D844 | A |
| 207 | Fregio curvo poppa Stern curved frieze Frieze courbe de poupe Heckkrummverzierung | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D845 | A |
| 208 | Parasartie di trinchetto Fore channel Porte-haubans de misaine Fockruste | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 209 | Parasartie di maestra Main channel Porte-haubans du grand mat Grossruste | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 210 | Parasartie di mezzana Mizzen channel Porte-haubans d'artimon Kreuzruste | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 211 | Bracciolo parasartie Channels knee Courbe des porte-haubans Riisteknie | massello chock bloc Block | 2 mm |  | P |
| 212 | Regolo parasartie Channels strip Lisse des porte-haubans Rusteband | listello faggio beech board ba guette hetre Buchenholzleiste | 2x2 mm |  | R |
| 213 | Albero di bompresso Bowsprit Beaupre Bugspriet | tondino ramino ramino round rond ramino Raminorund | d8 mm | TO 08 | R |
| 214 | Blocco manovre Rigging stop cleat Blocage pour manoeuvres Gutblokierung | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 215 | Bracciolo di bompresso - base Bowsprit knee - base Courbe de beaupre - base Bugsprietknie-Basis | listello faggio beech board baguette hetre Buchenholzleistt | 4x4 | LS257 | R |
| 216 | Bracciolo di bompresso -colombiere Bowsprit knee - head Courbe de beaupre - ton Bugsprietknie -Top | listello faggio beech board baguette hetre Buchenholzleist* | 4x4 | LS257 | R |
| 217 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 218 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 219 | Base coffa Top base Base de hune Marsbasis | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 220 | Rivestimento coffe Tops planking Revetement des hunes Marsenbeplankung | listello noce walnut board baguette noyer Nussholzleiste | 0,6x3 mm |  | R |
| 221 | Bordo coffe Tops rim Contour des hunes Marsenumrip | listello faggio beech board baguette hetre Buchenholzleiste | 1 x2 mm |  | R |
| 222 | Supporto corrimani coffe Tops handrails support Support des main courantes des hunes Marsenhandlaufenstiitze | massello chock bloc Block | 2 mm |  | P |
| 223 | Corrimano coffe Tops handrail Main courante des hunes Marsenhandlauf | list, faggio flex, beech board flex baguette hetre flex Buchenholzleiste flex | 2x2 mm | LF202 | R |
| 224 | Albero di parrocchetto di bompresso Bowsprit topmast Mat de perroquet de beaupre Sprietmast | listello faggio beech board baguette hetre Buchenholzleiste | 4x4 mm | LS257 | R |
| 225 | Chiave alberi Mastsfid Clefdes mats Schlopholz | listello faggio beech board baguette hetre Buchenholzleiste | 1x2 mm |  | R |
| 226 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T107 | P |
| 227 | Maschetta Cheek Jottereau Backe | listello faggio beech board baguette hetre Buchenholzleiste | 1 x3 mm |  | R |
| 228 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 1 x2 mm |  | R |
| 229 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleistt | 1 x1 mm |  | R |
| 230 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 1 x1 mm |  | R |
| 231 | Albero di pappafico di bompresso Bowsprit topgallant mast Fleche de beaupre Sprietflaggstenge | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm | - | R |
| 232 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T106 | P |
| 233 | Formaggetta Masts peak Pomme des mats Stengenknopf | legno wood bois Holz | d 4 mm |  | P |
| 234 | Albero di trinchetto Fore lower mast Bas mat de misaine Fockmast | tondino ramino ramino round rond ramino Raminorund | d 8 mm |  | R |
| 235 | Trinca Grammoning Rousture Wulinge | corda rope corde Sell | d 0,80 mm | C282 | C |
| 236 | Maschetta Cheek Jotterau Backe | listello faggio beech board baguette hetre Buchenholzleiste | 2x7 mm |  | R |
| 237 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 3x5 mm |  | R |
| 238 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 239 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 240 | Base coffa Top base Base de hune Marsbasis | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 241 | Rivestimento coffe Tops planking Rev§tement des hunes Marsenbeplankung | listello noce walnut board baguette noyer Nussholzleiste | 0,6x4 mm | LS225 | R |
| 242 | Supporto corrimani coffe Tops handrails support Support des main courantes des hunes MarsenhandlaufenstiJtze | massello chock bloc Block | 2 mm |  | P |
| 243 | Albero di parrocchetto Fore topmast Petit mat de hune Fockmarsstenge | listello faggio beech board baguette hetre Buchenholzleistf | 5x5 mm |  | R |
| 244 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T109 | P |
| 245 | Maschetta Cheek Jottereau Backe | listello faggio beech board baguette hetre Buchenholzleisti | 2x5 mm | LS259 | R |
| 246 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleistf | 2x3 mm |  | R |
| 247 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleist< | 2x2 mm |  | R |
| 248 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleisti | 2x2 mm |  | R |
| 249 | Base coffa Top base Base de hune Marsbasis | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 250 | Albero di velaccino Fore topgallant mast Petit mat de perroquet Fockbramstenge | listello faggio beech board baguette hetre Buchenholzleisti | 4x4 mm | LS257 | R |
| 251 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T107 | P |
| 252 | Maschetta Cheeck Jotterau Backe | listello faggio beech board baguette hetre Buchenholzleisti | 1 x3 mm |  | R |
| 253 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleisti | 1 x 2 mm |  | R |
| 254 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 1 x1 mm |  | R |
| 255 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 1x1 mm |  | R |
| 256 | Albero di controvelaccino Fore royal mast Fleche de misaine Fockflaggstenge | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 257 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T106 | P |
| 258 | Albero di maestra Main lower mast Grand mat Grossmast | tondino ramino ramino round rond ramino Raminorund | d 10mm | TO 10 | R |
| 259 | Trinca Gammoning Rousture Wulinge | corda rope corde sell | d 0,80 mm | C282 | C |
| 260 | Maschetta Cheek Jotterau Backe | listello faggio beech board baguette hetre Buchenholzleiste | 2x7 mm |  | R |
| 261 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 3x5 mm |  | R |
| 262 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 263 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 264 | Base coffa Top base Base de hune Marsbasis | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 265 | Albero di gabbia Main topmast Grand mat de hune Grossmarsstenge | listello faggio beech board baguette hetre Buchenholzleiste | 6x6 mm |  | R |
| 266 | Testa di moro Cap Chouquet Esalshaupt | legno wood bois Holz |  | T109 | P |
| 267 | Maschetta Cheek Jotterau Backe | listello faggio beech board baguette hetre Buchenholzleiste | 2x5 mm | LS259 | R |
| 268 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 2x3 mm |  | R |
| 269 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 270 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 271 | Base coffa Top base Base de hune Marsbasis | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 272 | Albero di velaccio Main topgallant mast Grand mat de perroquet Grossbramstenge | listello faggio beech board baguette hetre Buchenholzleiste | 4x4 mm | LS257 | R |
| 273 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T107 | P |
| 274 | Maschetta Cheeck Jotterau Backe | listello faggio beech board baguette hetre Buchenholzleiste | 1 x3 mm |  | R |
| 275 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 1 x2 mm |  | R |
| 276 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 1x1 mm |  | R |
| 277 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 1 x 1 mm |  | R |
| 278 | Albero di controvelaccio Main royal mast Fleche du grand mat Grossflaggstenge | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 279 | Testa di moro Cap Couquet Eselshaupt | legno wood bois Holz |  | T106 | P |
| 280 | Albero di mezzana Mizzen lower mast Bas mat d'artimon Kreuzmast | tondino ramino ramino round rond ramino Raminorund | d 6 mm | TO 06 | R |
| 281 | Maschetta Cheek Jotterau Backe | listello faggio beech board baguette hetre Buchenholzleiste | 2x5 mm | LS259 | R |
| 282 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 2x4 mm | LS258 | R |
| 283 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 284 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 285 | Base coffa Top base Base de hune Marsbasis | compensato plywood C.T.P. Sperrholz | 1,5 mm |  | P |
| 286 | Supporto corrimano coffa Top handrail support Support du main courante de hune Marshandlaufstutze | massello chock bloc Block | 2 mm |  | P |
| 287 | Albero di contromezzana Mizzen topmast Mat de perroquet de fouge Kreuzmarsstenge | listello faggio beech board baguette hetre Buchenholzleiste | 4x4 mm | LS257 | R |
| 288 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T108 | P |
| 289 | Maschetta Cheek Jotterau Backe | listello faggio beech board baguette hetre Buchenholzleiste | 1x3 mm |  | R |
| 290 | Barra costiera Trestle tree Elongis Langsaling | listello faggio beech board baguette hetre Buchenholzleiste | 1 x 2 mm |  | R |
| 291 | Barra traversa Cross tree Traversin Quersaling | listello faggio beech board baguette hetre Buchenholzleiste | 1x1 mm |  | R |
| 292 | Cuscino Bolster Coussin Kalben | listello faggio beech board baguette hetre Buchenholzleiste | 1 x1 mm |  | R |
| 293 | Albero di belvedere Mizzen topgallant mast Mat de perruche Kreuzbramstenge | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm |  | R |
| 294 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T106 | P |
| 295 | Bracciolo bandiera - base Flag knee - base Courbe du pavilion - base Flaggknie - Basis | listello faggio beech board baguette hetre Buchenholzleiste | 3x5 mm |  | R |
| 296 | Bracciolo bandiera - colonna Flag knee - pillar Courbe du pavilion - pilier Flaggknie - Pilaster | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 297 | Asta bandiera Flag mast Mat du pavilion Flaggmast | listello faggio beech board baguette hetre Buchenholzleiste | 3x3 mm | LS256 | R |
| 298 | Testa di moro Cap Chouquet Eselshaupt | legno wood bois Holz |  | T106 | P |
| 299 | Pennone di civada Spritsail yard Vergue de civadiere Blinderah | tondino ramino ramino round rond ramino Raminorund | d 4 mm | TO 04 | R |
| 300 | Pennone di controcivada Spritsail topsail yard Vergue du tourmentin Oberblinderah | tondino ramino ramino round rond ramino Raminorund | d 3 mm | TO 03 | R |
| 301 | Trevo di trinchetto Fore yard Vergue de misaine Fockrah | tondino ramino ramino round rond ramino Raminorund | d 6 mm | TO 06 | •R |
| 302 | Buttafuori trevo di trinchetto Fore yard stunsails boom Boute-hors de la vergue de misaine Fockrahleesegelspiere | tondino ramino ramino round rond ramino Raminorund | d 2 mm | TO 02 | R |
| 303 | Ferro per buttafuori Iron for stunsails booms Collier pour les boute-hors Leesegelspierenbrille | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | R |
| 304 | Trinca buttafuori Stunsails booms gammoning Rousture des boute-hors Leesegelspierenwulinge | corda rope corde Sell | d 0,50 mm | C281 | C |
| 305 | Pennone di parrocchetto Fore topsail yard Vergue de petit hunier Kockmarsrah | tondino ramino ramino round rond ramino Raminorund | d 4 mm | TO 04 | R |
| 306 | Pennone di velaccino Fore topgallant yard Vergue de petit perroquet Fockbramrah | tondino ramino ramino round rond ramino Raminorund | d 3 mm | TO 03 | R |
| 307 | Trevo di maestra Main yard Grand-vergue Grossrah | tondino ramino ramino round rond ramino Raminorund | d 8 mm | TO 08 | R |
| 308 | Buttafuori trevo di maestra Main yard stunsails boom Boute-hors de la grand-vergue Grossrahleesegelspiere | tondino ramino ramino round rond ramino Raminorund | d 3 mm | TO 03 | R |
| 309 | Ferro per buttafuori Iron for stunsails booms Collier pour les boute-hors Leesegelspierenbrille | ottone fotoinciso photoetched brass laiton photograve Photogravurmessing |  | D40 | R |
| 310d | Trinca buttafuori Stunsails booms gammoming Rousture des boute-hors Leesegelspierenwulinge | corda rope corde Sell | d 0,50 mm | C281 | C |
| 311 | Pennone di gabbia Main topsail yard Vergue de grand hunier Grossmarsrah | tondino ramino ramino round rond rarfiino Raminorund | d 5 mm | TO 05 | R |
| 312 | Pennone di velaccio Main topgallant yard Vergue de grand perroquet Grossbramrah | tondino ramino ramino round rond ramino Raminorund | d 3 mm | TO 03 | R |
| 313 | Antenna di mezzana Mizzen lateen yard Vergue d'artimon Kreuzrute | tondino ramino ramino round rond ramino Raminorund | d 6 mm | TO 06 | R |
| 314 | Guida per martinetto Bolster for lift Prise pour le martinet Dirkverbindung | listello faggio beech board baguette hetre Buchenholzleiste | 2x2 mm | - | R |
| 315 | Trinca guida martinetto Lift bolster gammoming Rousture de la prise du martinet Dirkverbindungwulinge | corda rope corde Sell | d 0,50 mm | C281 | C |
| 316 | Verga secca Mizzen yard Vergue seche Kreuzrah | tondino ramino ramino round rond ramino Raminorund | d 4 mm | TO 04 | R |
| 317 | Pennone di contromezzana Mizzen topsail yard Vergue de perroquet de fouge Kreuzmarsrah | tondino ramino ramino round rond ramino Raminorund | d 3 mm | TO 03 | R |
| 318 | Legatura o fasciatura su cavo Lashing or serving on rope Ligature ou fourrure du cordage Bindung Oder Seilwerkbund | corda rope corde Seil | d 0,25 mm | C280 | C |
| 319 | Legatura o fasciatura su cavo Lashing or serving on rope Ligature ou fourrure du cordage Bindung Oder Seilwerkbund | corda rope corde Seil | d 0,50 mm | C281 | C |
| 320 | Trinca bompresso Bowsprit gammoning Rousture du beaupre Sprietzurring | corda rope corde Seil | d 0,80 mm | C282 | C |
| 349d - 355d - 356d - 363d - 369d - 372d - 378d -379d - 380d - 387d - 393d - 396d - 402d - 403d -404d - 411d - 417d - 418d - 419d - 483 - 492 -501 - 506 - 508 - 513 - 518 - 520 - 525 - 540 -554d - 560d - 567d - 578 - 584d - 589d - 593d -655d - 676 - 681 d - 686d - 690d - 756d - 777 -782d - 787d - 791d - 818 - 822 - 839d - 871d -882 - 888d - 894d - 898d |  |  |  |  |  |
|  | Manovre Rigging Manoeuvres Tauwerk | corda rope corde Seil | d 0,25 mm | C280 | C |
| 329d - 339d - 351 d - 361 d - 370d - 373d - 377d -385d - 394d - 397d - 401 d - 409d - 41 2d - 41 6d -426d - 434d - 450 - 465 - 474 - 511 - 528 - 534 -548d - 550d - 564d - 571 - 572 - 585d - 603 -613d - 619d - 627d - 633d - 636 - 643 - 649d -651d - 665d - 669 - 670 - 682d - 700 - 711d -719d - 726d - 732d - 735 - 742 - 748d - 750d -766d - 770 - 771 - 783d - 794 - 800 - 813 - 825 -831 d - 833d - 850 - 858 - 865d - 867d - 875 -876 - 890d |  |  |  |  |  |
|  | Manovre Rigging Manoeuvres Tauwerk | corda rope corde Sell | d 0,50 mm | C281 | C |
| 341 d - 364d - 368d - 371 d - 388d - 392d - 395d -405d - 41 Od - 420d - 428d - 440 - 445 - 459 - 477 -486 - 495 - 502 - 596 - 597 - 608 - 61 5d - 629d -637 - 660d - 693 - 694 - 705 - 713d - 728d - 736 -761d - 807 - 843 - 844 - 852 |  |  |  |  |  |
|  | Manovre Rigging Manoeuvres Tauwerk | corda rope corde Sell | d 0,80 mm | C282 | C |
| 321d - 331d - 357d - 362d - 381d - 386d - 444 -446 - 451 - 468 - 605 - 702 |  |  |  |  |  |
|  | Manovre Rigging Manoeuvres Tauwerk | corda rope corde Seil | d 1,20 mm | C283 | C |
| 436 - 441 |  |  |  |  |  |
|  | Manovre Rigging Manoeuvres Tauwerk | corda rope corde Seil | d 2,00 mm | C284 | C |
| 352 - 353 - 374 - 375 - 398 - 399 - 413 - 414 |  |  |  |  |  |
|  | Bigotte Deadeyes Caps de mouton Jungfern | legno wood bois holz | d 3 mm | B80 | A |
| 358 - 359 - 365 - 366 - 382 - 383 - 389 - 390 -406 - 407 |  |  |  |  |  |
|  | Bigotte Deadeyes Caps de mouton Jungfern | legno wood bois holz | d 5 mm | B82 | A |
| 354 - 376 - 400 - 415 |  |  |  |  |  |
|  | Lande Deadeyes chains Cadenes Puttingeisen | filo ottone brass rod fil laiton Messingdraht | d 0,5 mm |  | R |
| 367 - 391 |  |  |  |  |  |
|  | Lande Deadeyes chains Cadenes Puttingeisen | ottone brunito browned brass laiton bruni Briiniertes Messing |  | (L60) | A |
| 360 - 384 |  |  |  |  |  |
|  | Lande Deadeyes chains Cadenes Puttingeisen | otone brunito browned brass laiotn bruni Bruniertes Messing |  | L93 | A |
| 408 | Landa Deadeye chain Cadene Puttingeisen | ottone brunito browned brass laiton bruni BriiniertesMessing |  | L92 | A |
| 325 - 335 - 422 - 430 |  |  |  |  |  |
|  | Lande modificate Modified deadeyes chains Cadenes modifiees Veranderndputtingeisen | ottone brunito browned brass laiton bruni BruniertesMessing |  | (L93) | R |
| 345 | Landa modificata Modified deadeye chain Cadene modifiee Veranderndputtingeisen | ottone brunito browned brass laiton bruni BruniertesMessing |  | (L92) | R |
| 514 - 516 - 521 - 523 - 535 - 537 - 562 - 565 -573 - 575 - 580 - 582 - 586 - 587 - 591 - 671 -673 - 677 - 679 - 683 - 684 - 688 - 772 - 774 -778 - 780 - 784 - 785 - 789 - 877 - 879 - 884 -886 - 891 - 892 - 896 |  |  |  |  |  |
|  | Bozzelli Blocks Poulies Blocke | legno 1 foro wood 1 hole bois 1 trou Holz 1 Loch | 3 mm | B100 | A |
| 460 - 462 - 503 - 504 - 509 - 530 - 532 - 543 -546 - 551 - 552 - 556 - 558 - 621 - 623 - 625 -645 - 647 - 652 - 653 - 661 - 663 - 720 - 722 -724 - 744 - 746 - 751 - 752 - 754 - 762 - 764 -808 - 810 - 819 - 823 - 827 - 829 - 834 - 836 -861 - 863 - 868 - 869 |  |  |  |  |  |
|  | Bozzelli Blocks Poulies Blocke | legno 1 foro wood 1 hole bois 1 trou Holz 1 Loch | 4 mm | B101 | A |
| 478 - 480 - 487 - 489 - 496 - 498 - 598 - 600 -695 - 697 - 795 - 797 - 845 - 847 - 853 - 855 - |  |  |  |  |  |
|  | Bozzelli Blocks Poulies Blocke | legno 2 fori wood 2 holes bois 2 trous Holz 2 Locher | 4 mm | B114 | A |
| 454 - 457 - 475 - 484 - 493 - 609 - 616 - 617 -631 - 640 - 658 - 707 - 714 - 717 - 730 - 759 -803 - 805 |  |  |  |  |  |
|  | Bozzelli Blocks Poulies Blocke | legno 1 foro wood 1 hole bois 1 trou Holz 1 Loch | 5 mm | B102 | A |
| 469 - 471 - 739 |  |  |  |  |  |
|  | Bozzelli Blocks Poulies Blocke | legno 2 fori wood 2 holes bois 2 trous Holz 2 Locher | 5 mm | B115 | A |
| 466 | Bozzello Block Poulie Block | legno 1 foro wood 1 hole bois 1 trou Holz 1 Loch | 7 mm | B104 | A |
| 447 - 448 - 607 |  |  |  |  |  |
|  | Bozzelli Blocks Poulies Blocke | legno 3 fori wood 3 holes bois 3 trous Holz 3 Locher | 7 mm | B130 | A |
| 437 - 438 - 442 - 443 |  |  |  |  |  |
|  | Bozzelli Blocks Poulies Blocke | legno 3 fori wood 3 holes bois 3 trous Holz 3 Locher | 10 mm | B132 | A |
| 322 - 326 - 332 - 336 - 342 - 346 - 421 - 423 -429 - 431 - 452 - 611 - 638 - 709 - 815 |  |  |  |  |  |
|  | Bozzelli violino Long tackle blocks Poulies a violon Violinblocke | legno 2 fori wood 2 holes bois 2 trous Holz 2 Locher | 8 mm | B140 | A |
| 606 - 703 - 737 |  |  |  |  |  |
|  | Bozzelli violino Long tackle blocks Poulies a violon Violinblocke | legno 3 fori wood 3 holes bois 3 trous Holz 3 Locher | 8 mm | B144 | A |
| 505 - 507 - 512 - 519 - 817 - 821 |  |  |  |  |  |
|  | Bozzelli violino modificati Modified long tackle blocks Poulies a violon modifiees Veranderndviolinblocke | legno 2 fori wood 2 holes bois 2 trous Holz 2 Locher | 8 mm | (B 140) | R |
| 344 - 348 - 482 - 491 - 500 - 515 - 517 - 522 -524 - 539 - 553 - 563 - 577 - 581 - 583 - 588 -592 - 654 - 675 - 678 - 680 - 685 - 689 - 753 -755 - 776 - 779 - 781 - 786 - 790 - 838 - 870 -881 - 885 - 887 - 893 - 897 |  |  |  |  |  |
|  | Stroppi bozzelli Blocks strops Estropes des poulies Blbckenstroppen | corda rope corde Sell | d 0,50 mm | C281 | C |
| 324 - 328 - 334 - 338 - 425 - 433 - 449 - 455 -458 - 464 - 473 - 476 - 485 - 494 - 510 - 531 -533 - 544 - 547 - 557 - 559 - 602 - 610 - 612 -618 - 622 - 624 - 626 - 632 - 642 - 646 - 648 -659 - 662 - 664 - 699 - 708 - 710 - 718 - 721 -723 - 725 - 731 - 741 - 745 - 747 - 760 - 763 -765 - 799 - 804 - 806 - 812 - 816 - 820 - 824 -828 - 830 - 849 - 857 - 862 - 864 |  |  |  |  |  |
|  | Stroppi bozzelli Blocks strops Estropes des poulies Blockenstroppen | corda rope corde Seil | d 0,80 mm | C282 | C |
| 439 - 467 |  |  |  |  |  |
|  | Stroppi bozzelli Blocks strops Estropes des poulies Blockenstroppen | corda rope corde Seil | d 1,20mm | C283 | C |
| 542 - 644 - 657 - 743 - 758 - 860 |  |  |  |  |  |
|  | Borrelli Toggles Guingonneaux Knebel | ottone brass laiton Messing |  | B250 | A |
| 453 - 456 - 461 - 470 - 479 - 488 - 497 - 529 -536 - 545 - 566 - 574 - 599 - 630 - 639 - 672 -696 - 704 - 715 - 716 - 729 - 738 - 773 - 796 -802 - 809 - 835 - 846 - 854 - 878 |  |  |  |  |  |
|  | Golfari Eyel bolts Pitons Augbolzen | ottone brass laiton Messing |  | A 98 | A |
| 323 - 327 - 333 - 337 - 343 - 347 - 424 - 432 - 463 -472 - 481 - 490 - 499 - 538 - 576 - 601 - 641 -674 - 698 - 740 - 775 - 798 - 811 - 837 - 848 -856 - 880 |  |  |  |  |  |
|  | Uncini Hooks Crocs Hacken | ottone brass laiton Messing |  | (A 98) | R |
| 527 - 570 - 595 - 635 - 668 - 692 - 734 - 769 -793 - 842 - 874 |  |  |  |  |  |
|  | Bertocci trozze Parrals trucks Pommels des racages Rackklotjes | vetro glass verre Glas |  |  | A |
| 526 - 569 - 634 - 667 - 733 - 768 - 792 - 841 -873 |  |  |  |  |  |
|  | Piastrine trozze Parrals ash ribs Bigots des racages Rackschlieten | ottone fotoinciso 2 fori photoetched brass 2 holes laiton phograve 2 trous Photogravurmessing 2 Locher |  | D40 | A |
| 594 - 691 |  |  |  |  |  |
|  | Piastrine trozze Parrals ash ribs Bigots des racages Rackschlieten | ottone fotoinciso 3 fori photoetched brass 3 holes laiton photograve 3 trous Photogravurmessing 3 Locher |  | D40 | A |
| 330 - 340 - 350 - 427 - 435 - 549 - 555 - 561 -590 - 614 - 620 - 628 - 650 - 656 - 666 - 687 -712 - 727 - 749 - 757 - 767 - 788 - 814 - 826 -832 - 866 - 872 - 889 - 895 - 899 |  |  |  |  |  |
|  | Caviglie Belayng pins Cabillots Belegnagel | legno wood bois Holz |  | C62 | A |
| 541 - 568 - 579 - 604 - 701 - 706 - 801 - 840 851 - 859 - 883 |  |  |  |  |  |
|  | Gallocce Cleats Taquets Klampen | legno wood bois Holz |  | G30 | P |
| 900 - 901 - 902 - 903 - 904 |  |  |  |  |  |
|  | Bandiere Flags Pavilions Flaggen | tessuto stampatc printed fabric tissu imprime Gedruckter Stoff |  | B340 | A |
| 905 | Canna tronca Half-guns Canon a piquer Halbrohr | ottone brunito browned brass Iaiton bruni Briiniertes Messing | 11 mm | C40 | A |
| 906 | Canna tronca Half-gun Canon a piquer Halbrohr | ottone brunito browned brass Iaiton bruni Bruniertes Messing | 14,5 mm | C43 | A |
| 907 | Canna tronca Half-gun Canon a piquer Halbrohr | ottone brunito browned brass Iaiton bruni Bruniertes Messing | 18 mm | C41 | A |
| 908 | Esterno mantelletti Ports lids outer plank Planche exterieure des mantelets Pfortendeckelauperplatte | listello noce walnut board baguette noyer Nussholzleiste | 1 x3 mm | LS230 | R |
| 909 | Interne mantelletti Ports lids inner plank Planche interieure des mantelets Pfortendeckelinnerplatte | listello faggio beech board baguette hetre Buchenholzleiste | 1 x3 mm |  | R |
| 910 | Esterno mantelletti Ports lids outer plank Planche exterieure des mantelets Pfortendeckelauperplatte | listello noce walnut board baguette noyer Nussholzleiste | 1x3 mm | LS230 | R |
| 911 | Interno mantelletti Ports lids inner plank Planche interieure des mantelets Pfortendeckelinnerplatte | listello faggio beech board baguette hetre Buchenholzleiste | 1 x3 mm |  | R |
| 912 | Golfare mantelletti Ports lids eyebolt Piton des mantelets Pfortendeckelaugbolzen | ottone brass Iaiton Messing |  | (A 98) | R |
| 913 | Drizza mantelletti Ports lids lanyard Drisse des mantelets Pfortendeckelfall | corda rope corde Seil | d 0,25 mm | C280 | C |
| 914 | Ritenuta mantelletti Ports lids retaining line Retenue des matelets Pfortendeckelverschlup | corda rope corde Seil | d 0,25 mm | C280 | C |
| 915 | Assale cannoni Guns axle Essieu des canons Kanonenachse | astina ottone brass rod barrette de laitor Messingstange | d 1,5 mm |  | R |
| 916 | Cannone Gun Canon Kanone | ottone brunito browned brass Iaiton bruni Bruniertes Messing | 25 mm | C31 | A |
| 917 | Affusto Carriage Affut Lafette | legno wood bois Holz | 16 mm | C31 | A |
| 918 | Ruota Truck Roue Rad | legno wood bois Holz | d 4 mm | C31 | A |
| 919 | Cannone Gun Canon Kanone | ottone brunito browned brass Iaiton bruni Bruniertes Messing | 30 mm | C32 | A |
| 920 | Affusto Carriage Affut Lafette | legno wood bois Holz | 19 mm | C32 | A |
| 921 | Ruota Truck Roue Rad | legno wood bois Holz | d 5 mm | C32 | A |
| 922 | Golfare brache cannoni Guns breechings eyebolt Piton des bragues des canons Kanonenbroktauaugbolzen | ottone brass laiton Messing |  | A 98 | A |
| 923 | Braca cannoni Guns breeching Brague des canons Kanonenbroktau | corda rope corde Sell | d 0,80 mm | C282 | C |
| 924 | Proiettile Shot Boulet Kugel | piombo lead plomb Blei |  |  | A |
| 925 | Porta proiettili Shots garland Port e-bou lets Kugelsull | corda rope corde Sell | d 0,80 mm | C282 | C |
| 926 | Barile polvere Powder barrel Baril de la poudre Pulverfass | legno wood bois Holz |  | B220 | A |
| 927 | Botte Cask Tonneau Tonne | legno wood bois Holz |  | B222 | A |
| 928 | Stelo fanali Lanthorns stalk Pivot des fanaux Laternenstander | astina ottone brass rod barrette du laiton Messingstange | d 1,5 mm |  | R |
| 929 | Cupola fanali Lanthorns dome Couple des fanaux Laternenkuppel | ottone brass laiton Messing |  | F2 | A |
| 930 | 1° settore vetrato 1st glazed sector 1er secteur vitre 1. Glassektor | materiale sintetico sinthetic material material sintetique Kunstoff |  | F2 | A |
| 931 | Telaio superiore Upper frame Chassis superieur Obergerust | ottone fotoinciso photoetched brass laiton photograv6 Photogravurmessing |  | F2 | A |
| 932 | 2° settore vetrato 2nd glazed sector 2eme secteur vitre 2. Glassektor | materiale sintetico sinthetic material material sintetique Kunstoff |  | F2 | A |
| 933 | Telaio inferiore Lower frame Chassis inferieur Untergeriist | ottone fotoinfciso photoetched brass laiton photograve Photogravurmessing |  | F2 | A |
| 934 | 3° settore vetrato 3rd glazed sector Seme secteur vitre 3. Glassektor | materiale sintetico sinthetic material material sintetique Kunstoff |  | F2 | A |
| 935 | Base fanali Lanthorns base Base des fanaux Laternenbasis | ottone brass laiton Messing |  | F2 | A |
| 936 | Spigolo fanali Lanthorns edge Arete des fanaux Laternenkante | filo ottone brass rod fil laiton Messingdraht | d 1,0 mm |  | R |
| 937 | Supporto fanali Lanthorns bracket Support des fanaux Laternenhalter | filo ottone brass rod fil laiton Messingdraht | d 0,5 mm |  | R |
| 938 | Ancora Anchor Ancre Anker | metallo brunito browned metal metal bruni Brunieries Metall | 55 mm |  | A |
| 939 | Ceppo ancore Anchors stock Jas des ancres Ankerstock | legno wood bois Holz | 55 mm |  | P |
| 940 | Anello ancore Anchors ring Organeau des ancres Ankerring | ottone brass laiton Messing | d 8 mm | A 94 | A |
| 941 | Boa ancore Anchors buoy Bouee des ancres Ankerboje | tondino ramino ramino round rond ramino Raminorund | d 6 mm | TO 06 | R |
| 942 | Imbracatura boe Buoys lashing Elingage des bouees Bojenanlegen | corda rope corde Sell | d 0,80 mm | C282 | C |
| 943d | Grippia boe Buoys rope Orin des bouees Bojenreep | corda rope corde Sell | d 0,50 mm | C281 | C |
| 944d | Cima boe Buoys line Vette des bouees Bojenlinie | corda rope corde Sell | d 0,25 mm | C280 | C |
| 945d | Cavo ancore Anchors rope Cable des ancres Ankertau | corda rope corde Seil | d 0,20 mm | C284 | C |
| 946 | Bozzello capone Cathead block Poulie de capon Kattaljeblock | legno 2 fori wood 2 holes bois 2 trous Holz 2 Locher | 4 mm | B114 | A |
| 947 | Stroppo Strop Estrope Stropp | corda rope corde Seil | d 0,80 mm | C282 | C |
| 948 | Bozzello capone Cathead block Poulie de capon Kattaljeblock | legno 2 fori wood 2 holes bois 2 trous Holz 2 Locher | 4 mm | B114 | A |
| 949 | Stroppo ad uncino Hook strop Estrope a croc Stropp mit Hacken | filo ottone brass rod fil laiton Messingdraht | d 1,0 mm |  | R |
| 950d | Cima paranchi capone Cathead tackles line Vette des palans de capon Kattaljelinie | corda rope corde Seil | d 0,50 mm | C281 | C |
| 951 | Galloccia capone Cathead cleat Taquet de capon Kattaljeklampe | legno wood bois Holz |  | G30 | P |
| 952d | Serrabozze (gru capone) Shank painter (catheads) Serre-bosses (bossoirs de capon) Riistleine (Kranbalken) | corda rope corde Seil | d 0,80 mm | C282 | C |
| 953d | Serrabozze (parasartie) Shank painter (channels) Serre-bosses (porte-haubans) Rustleine (Rusten) | corda rope corde Seil | d 0,80 mm | C282 | C |
| 954 | Braca timone Rudder breeching Brague du gouvernail Ruderbroktau | corda rope corde Seil | d 0,80 mm | C282 | C |
| 955 | Golfare trozza Slings eyebolt Piton des sauvegardes Sorgleinenaugbolzen | ottone brass laiton Messing |  | A 98 | A |
| 956d | Trozza timone Rudder sling Sauvegarde du gouvernail Rudersorgleine | corda rope corde Seil | 0 0,50 mm | C281 | C |
| 957 | Bozzello trozza Slings block Poulie des sauvegardes Sorgleinenblock | legno 1 foro wood 1 hole bois 1 trou Holz 1 Lock | 3 mm | B100 | A |
| 958 | Golfare cime trozza Slings lines eyebolt Piton des vettes des sauvegardes Sorgleinentaljenaugbolzen | ottone brass laiton Messing |  | A 98 | A |
| 959d | Cima trozza Slings line Vette des sauvegardes Sorgleinentalje | corda rope corde Seil | d 0,25 mm | C280 | C |
| 960 | Basamento Base Socle Grundbrett | massello chock bloc Block | 7 mm |  | P |
| 961 | Forcella centrale Midship saddle Berceau du milieu Zentralstander | compensato plywood C.T.P. Sperrholz | 5 mm |  | P |
| 962 | Supporto estremita Bow and stern fork Support des extremites Endenklampe | massello chock bloc Block | 10 mm |  | P |
| 963 | Targhetta Plate Plaque Schilder | fusione metallo dorato gilded cast metal fusion metal dore Metallguss Vergoldete |  | D232 | A |

SYMBOLS

A = Fitting (ready to assemble)

C = Rope

P = Premanufactured

R = Obtaining from...